BLE Zone Reader

Hardware BOM — Station Infrastructure

Pair Eyewear Irvine Labs  ·  32 Station Readers  ·  March 1, 2026

| 32 Station Readers | 10 Zones Covered | ~$2,800 Est. Hardware Total | –$6,700 vs. UWB alternative |
| --- | --- | --- | --- |

# How Zone Detection Works

A $15 Raspberry Pi at each station listens for BLE beacons on your trays. When a beacon signal exceeds -75 dBm, that tray is at that station. Zone event posts to Lab_Assistant automatically — no operator action.

Each station reader knows its own location. When it sees a beacon stronger than the RSSI threshold, it fires: {tray: "T-047", zone: "COATING-M1", event: "ENTER", time: now}. Lab_Assistant logs it, starts the dwell timer, and updates the floor map. When the tray leaves, RSSI drops below threshold — EXIT event fires, dwell time is recorded.

- Metal equipment and close quarters help — natural signal boundaries between stations. Coating machines act as RF shields.

- Coverage per reader: 3–6 meters — sufficient for zone detection at each machine or bench cluster.

- Each Pi runs a Node.js script — scans BLE every 500ms, applies RSSI threshold, POSTs events to Lab_Assistant server on WiFi.

- No new network infrastructure — readers connect to your existing UniFi WiFi. Assign static IPs via UniFi DHCP.

# Station Reader Map — Your Actual Floor

Based on your lab layout: Put Wall → Surfacing (7 stations) → Coating Line (Dip, Oven, 3 Coaters) → Edging (6 machines) → Assembly → QC → Ship. Total: 32 readers including 5 spare.

| Zone | Stations Covered | Readers | Pi Zeros | Install Notes |
| --- | --- | --- | --- | --- |
| Put Wall / Picking | Put Wall 1, Put Wall 2 | 2 | 2 | One reader per wall, mounted center-top facing down |
| Surfacing | Taper Blocker, Generator, Polisher, Laser, Deblock, Detape, Cleaning | 7 | 7 | One per machine — tight zones, metal boundaries help separation |
| Coating — Dip/Oven | Dip Station, Coating Oven | 2 | 2 | Two distinct stations, one reader each |
| Coating — Coater 1 | Coater 1 | 1 | 1 | Tablet already covers this zone — reader confirms tray entry/exit |
| Coating — Coater 2 | Coater 2 | 1 | 1 |  |
| Coating — Coater 3 | Coater 3 (Large) | 2 | 2 | Large coater = 2 readers for full coverage end-to-end |
| Cutting / Edging | Edger 1, Edger 2, Edger 3, Edger 4, Edger 5, Edger 6 | 6 | 6 | One reader per edger — each machine is a distinct zone |
| Assembly / Insert | Assembly Bench Row A, Row B | 3 | 3 | One per bench cluster (2–3 benches per reader at 4–5m range) |
| QC / Inspection | QC Station 1, QC Station 2 | 2 | 2 | One per inspector position |
| Ship / Dispatch | Dispatch / Ship Station | 1 | 1 | Single reader covers full dispatch area |
| SPARE | — | 5 | 5 | 2 per site (Irvine 1 + Irvine 2) + 1 general spare |
| TOTAL READERS / Pi ZEROS |  | 32 | 32 | Adjust surfacing + assembly counts to match actual floor |

⚠ Adjust assembly bench cluster count to match your actual floor. One reader covers 2–3 benches at 4–5m range. If benches are spread far apart, add readers.

# Bill of Materials

① Raspberry Pi Zero 2W Kits — One Per Station

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| Raspberry Pi Zero 2W Main compute — runs Node.js BLE scanner, posts events to server | Quad-core 64-bit ARM, 512MB, WiFi | 32 | $15 | $480 | Adafruit — Pi Zero 2W Check stock — also PiShop.us and Vilros.com. Sells out — order early. |
| MicroSD Card 32GB — Samsung Endurance OS storage — 24/7 rated write cycles | Samsung PRO Endurance 32GB microSD | 32 | $9 | $288 | Amazon — Samsung PRO Endurance 32GB Endurance series only — regular MicroSD wears out in months at continuous write. |
| 5V 2.5A Micro-USB Power Supply Power each Pi Zero at its station | CanaKit 5V 2.5A micro-USB — UL listed | 32 | $8 | $256 | Amazon — CanaKit Raspberry Pi PSU Or run PoE HAT + PoE switch if you prefer one cable per station. |

| SUBTOTAL — Pi Zero Kits (32 units) | $1,024 |
| --- | --- |

② USB BLE 5.0 Adapters — One Per Reader

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| ASUS USB-BT500 BLE 5.0 Adapter External antenna — measurably better range and stability than Pi built-in | BLE 5.0, USB-A, external antenna, Broadcom chip | 32 | $18 | $576 | Amazon — ASUS USB-BT500 External antenna is worth the extra $8 vs built-in. Zone boundary is sharper. |
| USB-A to Micro-USB OTG Adapter Pi Zero 2W has micro-USB only — need OTG to plug in USB BLE adapter | USB-A female to micro-USB male OTG | 32 | $1 | $32 | Amazon — micro USB OTG adapter 10-pack Buy a 10-pack — cheap and easy to lose during setup. |

| SUBTOTAL — BLE Adapters + OTG (32 units) | $608 |
| --- | --- |

③ Enclosures + Mounting — Protect Each Pi at Workstation

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| IP54 Project Enclosure for Pi Zero Protects from lens coating spray, dust, and cleaning chemicals | Hammond 1591XXFLBK or equiv — ~115×65×40mm ABS | 32 | $8 | $256 | Amazon — Pi Zero project box enclosure Drill one hole for power cable, small slot for BLE adapter. No need to be airtight. |
| 3M Command Large Strips Mount enclosures at station — no drilling into equipment or walls | 3M Command Large strips — 5 lb hold each | 3 | $14 | $42 | Amazon — 3M Command large strips 3 packs covers all 32 readers with spares. Removable if you need to reposition. |
| Velcro Heavy Duty (backup mounting) For stations where Command strips won't bond to surface | 3M Industrial Velcro — 2" width | 1 | $14 | $14 | Amazon — 3M heavy duty velcro Use on metal machine surfaces where Command strips underperform. |

| SUBTOTAL — Enclosures + Mounting | $312 |
| --- | --- |

④ Network — Zero Additional Cost

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| WiFi — Existing UniFi Network Each Pi Zero 2W connects to your existing lab WiFi — no new infrastructure | 802.11 b/g/n 2.4GHz built-in — connects to existing SSID | 1 | $0 | $0 | Already have it — UniFi Create a dedicated IoT VLAN in UniFi if security policy requires device isolation. |
| Static IP Reservations in UniFi Reserve a static IP per Pi by MAC address — keeps reader config stable | UniFi DHCP static mapping — 30 min setup | 32 | $0 | $0 | UniFi controller — already installed Do this before mounting readers. Bind each Pi MAC to a fixed IP during bench setup. |

| SUBTOTAL — NETWORK | $0 (existing infrastructure) |
| --- | --- |

⑤ One-Time Setup Tools

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| MicroSD Card Reader/Writer Flash Raspberry Pi OS Lite onto each card before first boot | USB-A + USB-C combo reader | 1 | $10 | $10 | Amazon — USB microSD reader Use Raspberry Pi Imager (free). Pre-configure WiFi + SSH in Imager — no keyboard needed after flash. |
| Mini HDMI to HDMI Adapter First-boot only if SSH pre-config fails — plug into any monitor | Mini HDMI male to HDMI female | 1 | $8 | $8 | Amazon — mini HDMI adapter One adapter serves entire fleet setup. Only needed if you can't SSH on first boot. |

| SUBTOTAL — SETUP TOOLS | ~$18 |
| --- | --- |

| TOTAL BLE ZONE READER BOM — 32 STATIONS | ~$2,800 |
| --- | --- |

vs. UWB RTLS alternative for same coverage: ~$9,500 SAVINGS: ~$6,700 Same zone-level operational visibility.

# Pilot Order — Validate One Coating Zone First

Order this before committing to the full BOM. One coating machine, one week of data, validates everything before you spend $2,800.

| Item | Qty | Cost | Purpose |
| --- | --- | --- | --- |
| Raspberry Pi Zero 2W | 2 | $30 | One reader at Coating M1, one spare |
| Samsung Endurance 32GB microSD | 2 | $18 | OS for both Pis |
| ASUS USB-BT500 BLE adapter | 2 | $36 | One per Pi |
| Micro-USB OTG adapter | 2 | $2 | Pi Zero OTG port |
| 5V 2.5A power supply | 2 | $16 | Power both Pis |
| Project enclosure | 2 | $16 | Mount at machine |
| 3M Command strips (1 pack) | 1 | $14 | Mount enclosures |
| MicroSD reader + mini HDMI | 1 | $18 | Setup tools |

| PILOT ORDER TOTAL | ~$150 |
| --- | --- |

# Setup Steps Once Hardware Arrives

- Flash each microSD — Raspberry Pi Imager (free). Pre-configure your WiFi SSID/password and enable SSH in Imager settings before writing. No keyboard or monitor needed after this.

- Assemble each Pi — insert SD, plug OTG → BLE adapter, mount in enclosure, cable through hole.

- Assign static IPs — in UniFi DHCP, bind each Pi MAC address to a fixed IP before powering on. Simplifies reader config.

- SSH into each Pi — install Node.js: `curl -fsSL https://deb.nodesource.com/setup_20.x | sudo -E bash - && sudo apt install nodejs`. Deploy the reader script.

- Mount enclosures — Command strips or Velcro at each station. Ceiling or wall mount at 2–2.5m height, antenna pointing down toward tray level.

- Walk calibration — carry a tray beacon through each zone. Adjust RSSI threshold in reader config until zone boundary lands exactly where you want it. Target: tray detects within 0.5m of machine, drops out within 1m of leaving.

- Monitor in Lab_Assistant — zone events appear in real-time on the floor map. Run 1 week on pilot zone, validate dwell times against operator notes. Expand to all stations.

Prices estimated March 1, 2026 — verify before ordering. Coater 3 gets 2 readers due to larger footprint. Surfacing has 7 readers (one per machine) — adjust if machine count differs. Assembly cluster count assumes 3 bench groups. Spare readers: 5 total (2 per Irvine site + 1 float). Pair Eyewear internal use only.