Smart Tray Retrofit

Phase 1 BOM — Beacon + QR Label on Existing Trays

Pair Eyewear Irvine Labs  ·  120 Trays + 20 Spare  ·  March 1, 2026

| 120 Trays (production) | 20 Spare stock | ~$12 Cost per tray | ~$1,700 Total BOM est. |
| --- | --- | --- | --- |

# Scope — Retrofit Only

Existing trays stay as-is. We add two things: a BLE beacon (zone tracking) and a QR label (scan-to-bind). Two minutes per tray. One assembly day for the full fleet.

|  | Component | Function | Time to install | Cost/tray |
| --- | --- | --- | --- | --- |
| ① | BLE Beacon (Minew E7) | Zone detection — tray broadcasts ID to station readers | 45 sec | $11 |
| ② | QR Code Label (laminated) | Scan-to-bind at put wall and quick bind stations | 30 sec | $0.40 |
|  | 3M VHB pad + IPA wipe | Mounting consumables for beacon attachment | 15 sec | $0.20 |

# Bill of Materials

① BLE Beacon Tags — Minew E7 (One Per Tray)

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| Minew E7 BLE 5.0 Beacon Primary recommendation — IP67, stable RSSI, 18-month battery | BLE 5.0, CR2032, 30×30×8mm, IP67 | 140 | $11 | $1,540 | Minew AliExpress Store 120 prod + 20 spare. Email sales@minewsemi.com — expect $9–10 at 140 units |
| Minew E7 — US stock alternative Ships faster, higher unit price | Same SKU — MinewSemi.com US warehouse | 0 | $13 | see note | minewsemi.com Use if AliExpress 3–4 week lead time is a problem. US ships in 3–5 days. |
| Feasycom FS-BT0904A (backup) If E7 out of stock — similar spec, same form factor | BLE 5.0, IP67, CR2032, configurable interval | 0 | $9 | see note | Feasycom.com or AliExpress Acceptable substitute. Slightly smaller — same VHB mount works fine. |
| CR2032 Battery — Annual spare stock Replace all beacons same day each year — buy one full cycle now | Panasonic CR2032 industrial — 3V | 140 | $0.60 | $84 | Amazon — Panasonic CR2032 100-pack Schedule annual swap in Q1 PM calendar. Takes 30 min for 120 trays. |

| SUBTOTAL — BLE BEACONS (140 units + battery spares) | ~$1,624 |
| --- | --- |

② QR Code Labels — Laminated Polyester (One Per Tray)

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| Brother QL-820NWB Label Printer Prints QR labels on demand — same printer for tray and put wall labels | 62mm DK tape, WiFi + USB, 300 dpi | 1 | $130 | $130 | Amazon — Brother QL-820NWB One printer serves tray labels, put wall QR sheets, and job tickets. Already in BLE BOM. |
| White DK-2205 Label Tape — 62mm Print substrate for QR labels | Brother DK-2205, continuous 62mm roll, 30.48m | 3 | $14 | $42 | Amazon — Brother DK-2205 One roll = ~240 labels at 40mm each. 3 rolls = fleet + spares + reprints. |
| Clear Laminate Tape — 62mm Overlaminate each label — coating chemistry destroys unprotected labels in days | Compatible clear DK laminate or thermal overlaminate roll | 3 | $14 | $42 | Amazon — Brother compatible clear laminate 62mm Non-negotiable in a coating lab. Laminated labels last 18+ months. |
| QR Code Generation Generate tray ID QR codes T-001 through T-140 | Lab_Assistant built-in QR generator (already built) | 1 | $0 | $0 | Lab_Assistant — Smart Trays → QR Generator tab Exports print-ready layout. Or use qr-code-generator.com for individual codes. |

| SUBTOTAL — QR LABELS + PRINTER | ~$214 |
| --- | --- |

③ Mounting Consumables — Attach Beacon to Tray

| Component | Spec / Model | Qty | Unit $ | Total | Source / Notes |
| --- | --- | --- | --- | --- | --- |
| 3M VHB 4910 Tape — 1" roll Mounts beacon to tray. Chemical-resistant, permanent bond. | 3M VHB 4910, 1" × 36 yard roll, clear | 1 | $22 | $22 | Amazon — 3M VHB 4910 1 inch tape One roll does 300+ beacon mounts. Pre-cut 30×30mm pads before assembly. |
| IPA Wipes — pre-saturated Clean tray surface before beacon attachment — critical for adhesion | 70% IPA wipes, individually sealed, 100-pack | 1 | $12 | $12 | Amazon — isopropyl alcohol wipes 100 pack One wipe per tray. Do not skip — VHB will not bond to contaminated surface. |
| 120-grit Sandpaper (optional) Scuff glossy tray surfaces before VHB for better adhesion | 3M 120-grit, half-sheet pack | 1 | $8 | $8 | Amazon or Home Depot Only needed if existing trays have smooth/glossy surfaces. Skip for textured. |

| SUBTOTAL — MOUNTING CONSUMABLES | ~$42 |
| --- | --- |

| TOTAL RETROFIT BOM — 140 TRAYS (120 production + 20 spare) | ~$1,880 |
| --- | --- |

~$12/tray all-in. 2 minutes per tray. One person does the full fleet in a single day.

# Start With a Pilot Order — 10 Trays

Validate beacon RSSI and zone boundary before committing to 140 units. Pilot order is ~$130 and takes one afternoon.

| Item | Qty | Cost | Purpose |
| --- | --- | --- | --- |
| Minew E7 Beacons | 10 | $110 | Pilot 10 trays through one coating zone |
| 3M VHB tape (small roll) | 1 | $8 | Mount 10 beacons + spares |
| IPA wipes | 1pk | $12 | Surface prep |
| Label tape roll | 1 | $14 | Print pilot QR labels |

| PILOT ORDER TOTAL | ~$144 |
| --- | --- |

Run 10 trays through the coating zone for one week. Confirm beacon detection events appear in Lab_Assistant, zone boundary is clean, and RSSI thresholds are dialed in. Then order the remaining 130.

# Assembly — 7-Step Process Per Tray

Target: 2 minutes per tray with a prepped station. One person can retrofit the full 120-tray fleet in a single 4-hour session.

| Step | Operation | How | Field Note |
| --- | --- | --- | --- |
| 1 | Clean tray surface | IPA wipe on beacon mount zone (rear top edge — clear of lens surface) | 30 seconds. Must be clean and dry before VHB. Skip this and beacon falls off in a week. |
| 2 | Scuff mount zone | 120-grit sand ~35×35mm patch where beacon will sit | Only if surface is very smooth/glossy. Skip for textured tray surfaces. |
| 3 | Cut VHB pad | Pre-cut 30×30mm pad from 3M VHB 4910 roll | Pre-cut a full batch of pads before assembly day — faster on the line. |
| 4 | Attach BLE beacon | Peel VHB, press Minew E7 firmly for 30 seconds | Beacon goes rear-top edge of tray. Away from lens area. Antenna pointing up. |
| 5 | Apply QR label | Peel and stick laminated QR label — adjacent to beacon | Label encodes tray ID: T-001 through T-120. Print from Lab_Assistant QR generator. |
| 6 | Test beacon | Scan with Lab_Assistant BLE test or phone NRF Connect app | Confirm tray ID broadcasts correctly before registering in fleet. |
| 7 | Register in fleet | Scan tray QR in Lab_Assistant → add to fleet → state = IDLE | Takes 10 seconds. Confirm beacon shows in position map within 60 seconds of deployment. |

## Assembly Station Setup

- Label printer — Brother QL-820NWB connected to laptop, QR template loaded

- VHB pads — pre-cut 30×30mm stack of 140 pads the day before (45 min, do it once)

- IPA wipes — one per tray, open as you go

- Rubber roller or thumb — press beacon for 30 seconds after placement

- Phone with NRF Connect app — spot-check beacon broadcasts during assembly

- Laptop with Lab_Assistant — register each tray immediately after attaching

## Ordering Sequence

- Order pilot (10 beacons) — place today, arrives in 3–4 weeks from AliExpress or 3–5 days from MinewSemi US

- Run pilot — 10 trays through coating zone, validate detection for 1 week

- Dial in RSSI threshold — adjust reader config until zone boundary is exactly where you want it

- Order remaining 130 beacons — email MinewSemi for bulk quote at 130 units

- Assembly day — retrofit all 140 trays in one session, register fleet, deploy

# What Phase 1 Enables — Day One

Once 140 trays are retrofitted and the BLE station readers are installed:

- Zone visibility — every tray location known in real-time: Coating M1, Edger 3, QC Station 2

- Dwell time — automatic timer from zone entry to exit, no operator action

- Job routing history — complete path of every job through the lab

- Operator attribution — who touched what tray and when

- AI queries — "Where is job J21694?" answered in <1 second from Lab_Assistant

- Coating correlation — which jobs were in M2 during the bad batch on Tuesday

Phase 2 upgrade path: Add NFC sticker to each tray at next maintenance cycle — no beacon removal, just add sticker and seal. Tap-to-bind replaces scan-to-bind. No other tray rework needed.

Prices estimated March 1, 2026 — verify before ordering. Quantities: 120 production + 20 spare. Beacon choice Minew E7 — substitute Feasycom FS-BT0904A if out of stock. Label printer (Brother QL-820NWB) shared with put wall QR sheet printing and job ticket printing. Pair Eyewear internal use only.